# HackMIT 2026 Hardware Setup

Webcam interaction and two display demo

Updated September 19, 2026. The GX10 runs all models and the app. The ASUS portable monitor shows the physics simulation; the 7-inch touchscreen provides the menu and Take a picture control. One webcam handles both printed-problem capture and hand tracking.

# Active hardware checklist

| Qty | Hardware | Purpose and check |
| --- | --- | --- |
| 1 | ASUS Ascent GX10 | Compute host. Include its power supply; Claude has configured local inference and the desktop runtime. |
| 1 | ASUS portable monitor | Simulation, equations and graphs. Confirm exact model, video inputs and power needs from its label. |
| 1 | 7-inch touchscreen | Capture menu and status. Verify video, power and touch-data connections. |
| 1 | Logitech C270 webcam | USB connection to GX10. Test readable printed problems and hand tracking in the same setup. |
| 1 ea. | Keyboard and mouse | Setup, window placement and recovery. |
| Set | Display and USB cables | Match actual device ports. Verify two simultaneous video outputs; a USB hub is not automatically a video adapter. |
| Set | Stands and cable restraints | Secure both screens and the webcam. Mark repeatable page and hand positions. |

# Confirmed demo scope

Four problem types: projectile, inclined plane, pendulum and 1D collision. The visitor taps Take a picture, frames the problem, and loads its matching simulation on the ASUS screen. After capture, remove the page and resume hand interaction.

Set aside the previous depth sensors, BNO055, ESP32 or UNO Q, breadboards, servo hardware and haptics. No sensor firmware, serial bridge, soldering or depth calibration is required for this demo.

# Assembly and verification

Check the ASUS monitor model and ports, then establish a working connection for both displays to the GX10. Use an extended desktop.

Map touch input to the 7-inch screen. Check all four corners, rotation and touch accuracy without affecting the ASUS display.

Mount the webcam and test printed text under table lighting. Confirm the same camera can see the hand comfortably after the page is removed.

Open the app on GX10. When the two views are implemented, place the controller on the touchscreen and the simulation on the ASUS monitor.

Test one printed example of each confirmed problem type. Verify local extraction with no hosted API key; show loading, retry and unsupported-input states.

Test capture-to-hand switching, tracking loss and recovery. Capture mode must pause interaction; hand reacquisition must not throw an object using stale motion.

Restart and repeat the full flow. Record a working demo video and keep prepared problems available if photo capture fails.

# Implementation and evidence

Claude reported working GX10 local photo extraction in about 28 seconds on a synthetic problem image. That is not yet validation of all four printed examples or the two-display user flow.

The touchscreen capture menu, shared controller and simulation state, and real webcam HandSource integration remain software tasks. Existing simulation and extraction code does not by itself provide this complete flow.

# Open hardware checks

Earlier inventory calls the ASUS monitor a ZenScreen; the user referred to it as a Zenbook portable monitor. Confirm its exact label before selecting cables. Simultaneous GX10 video output, touchscreen mapping and camera placement still need physical testing.

# References

hackmit-2026-plan.md is the current project plan. gx10/README.md contains deployment details. HackMIT 2026 Hardware List.pdf is the original organizer inventory catalog and remains unchanged; its sensors and motors are not project requirements.